Reference: MRA 1

North Sunderland Harbour Risk Assessment Form

| Service | North Sunderland Harbour - MRA Number 1 | Date of Risk Assessment | 2nd April 2026 |
| --- | --- | --- | --- |
| Assessor | Fiona Kibby |  |  |
| Process or Activity | Slips, Trips and Falls at the Quayside |  |  |

| Describe the location where activities are taking place | North Sunderland Harbour quays, jetties, pontoons and slipways. |  |  |
| --- | --- | --- | --- |
| Describe the activities being undertaken | Harbour users accessing quaysides, ascending or descending ladders, using slipways, pontoons or quayside ladders to access vessels. |  |  |
| Describe the equipment and/ or substances being used | North Sunderland Harbour ladders, slipways, pontoon and quayside. |  |  |
| Describe the people involved in these activities (and others who might be affected) | Harbour Staff, commercial fishermen, tripping boat crew, divers, RNLI crew, leisure users and members of the public. |  |  |
| Associated Risk Assessments | MRA 2 / MRA 9 / MRA 11 | Associated Safe Systems of Work | Port Marine Safety Code. Guide to Good Practice. NSH maintenance programme. |

| Risk Rating | Action |
| --- | --- |
| 20-25 | Stop Activity and Take Immediate Action |
| 15-20 | Urgent Action |
| 8-12 | Action |
| 3-6 | Monitor |
| 1 -2 | No Action – ensure controls are maintained and reviewed |

| Hazard | Uncontrolled Risk Score |  |  | Risk Mitigations | Residual Risk Score |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | L | C | R |  | L | C | R |
| Harbour staff falls from NSH Quays at height into the water. | 2 | 4 | 8 | - All harbour staff are advised to self declare their fitness to work and not attend work if unable to work safely. - All harbour staff wear safety footwear. - All harbour staff wear 150N lifejackets when working on pontoon, directly above or on water. - Harbour walls have prominent “do not climb” signs on them. - Guardrail and Armco barriers are surveyed 2 yearly and a regular programme of maintenance & replacement in place. - Harbour staff are first aid trained. - Harbour lifebuoys are surveyed and inspected monthly. - Harbour staff have appropriate PPE including safety boots rated to ISO 20345 standard. | 2 | 2 | 4 |
| Contractor slips or falls from height on harbour property | 2 | 4 | 8 | - Contractors are required to provide their own risk assessments which are reviewed by Harbour Master. - Contractors must notify harbour staff of their arrival and departure from site. - Contractors can be provided with 150N lifejackets when working on pontoon, directly above or on water if they don’t have their own. - Guardrail and Armco barriers are surveyed 2 yearly and a regular programme of maintenance & replacement in place. - Harbour staff and commercial users are first aid trained - Harbour lifebuoys are surveyed and inspected regularly - Harbour staff are on duty during normal office hours - Ladders, slipways, steps and pontoon are cleaned by harbour staff regularly to avoid marine growth. - Ladders are surveyed 2 yearly and a regular programme of maintenance & replacement in place. | 2 | 2 | 4 |
| Member of public falls from NSH Quays at height into water. | 3 | 4 | 12 | - High quay walls and strong guardrails discourage all but the nimblest from climbing over. - Harbour walls have prominent “do not climb” signs on them. - Guardrail and Armco barriers are surveyed 2 yearly and a regular programme of maintenance & replacement in place. - Harbour lifebuoys are surveyed and inspected monthly. - Harbour staff are on site during normal office working hours. - Harbour CCTV system will usually spot miscreants. - “No swimming” Byelaw for harbour waters. | 2 | 3 | 6 |
| Harbour user falls from ladder. | 3 | 4 | 12 | - Harbour staff are advised to self declare their fitness to work and not attend work if unable to work safely. - All harbour staff wear safety footwear. - All harbour staff and commercial users wear lifejackets when working on pontoons, directly above or on water. - Ladders are cleaned by harbour staff regularly to avoid marine growth. - Ladders are surveyed 2 yearly and a regular programme of maintenance & replacement in place. - Harbour lifebuoys are surveyed and inspected monthly. - Harbour staff and commercial users are first aid trained. | 2 | 3 | 6 |
| Passenger slips / falls on commercial public embarkation slipway / pontoon/ steps. | 4 | 4 | 16 | - Tripping boat risk assessments and operating procedures are reviewed each season. - Slipway is cleaned / jet washed regularly in summer months by harbour staff. Monthly in winter. - Harbour boarding steps are cleaned by harbourstaff regularly to avoid marine growth. - Harbour boarding steps are surveyed 2 yearly and a regular programme of maintenance & replacement in place. - Pontoon is cleaned to avoid marine growth and a regular programme of survey and maintenance to check for slip/trip hazards. - Prior to embarkation each passenger is scrutinized for physical ability and assistance offered if required for boarding. - Prior to going onto slipway / pontoon passengers are briefed by boat crew. - Passengers for embarkation in trip boats are closely supervised by boat crew. - Slipway is not used 1 hour each side of low water (non slippy). | 2 | 4 | 8 |
| Passenger falls between vessel and slipway or pontoon while embarking. | 4 | 5 | 20 | - Tripping boat risk assessment and operating procedures are reviewed by HM for each vessel. - Slipway is cleaned / jet washed frequently in summer months by harbour staff. Monthly in winter. - Prior to embarkation each passenger is scrutinized for physical ability and assistance offered if required for boarding. - Prior to going onto slipway or pontoon passengers are briefed by boat crew. - Embarkation process uses 2 crew members to hand passengers on board - one on vessel and one on slipway. - Passengers must disembark on pontoon and slipway before next group embark to avoid crowding. - When gap between vessel and slipway is more than 50cm, a gangway / boarding plank is used. - Slipway is not used 1 hour each side of low water (non slippy). | 1 | 4 | 4 |
| Harbour user slips on public launching slipway or pontoon. | 3 | 4 | 12 | - Slipway /pontoon is cleaned / jet washed frequently by harbour staff. - Public access to public launching slipway and pontoon is controlled by lock and keys – harbour staff. | 1 | 2 | 2 |
| Diver / Fisherman slips on boarding steps while loading / embarking. | 3 | 4 | 12 | - Harbour boarding steps are cleaned by harbour staff regularly to avoid marine growth. - Harbour boarding steps are surveyed 2 yearly and a regular programme of maintenance & replacement in place. | 2 | 3 | 6 |
| RNLI crew slip during launch or recovery. | 4 | 4 | 16 | - RNLI risk assessment and operating procedures for launch are reviewed by Harbour Master. - Slipway is cleaned / jet washed frequently in summer months by harbour staff. Monthly in winter. - Each launch is approved by RNLI launch authority who conduct dynamic risk assessment. - All RNLI staff follow RNLI SOPS for launch. | 3 | 3 | 9 |